# Required Components

|  | BOM - Gazillion Tornado Bubble Machine - 3.5 mm Mono jack - 2x 8 cm lengths of wire - 4x AA batteries (Not shown in picture) |
| --- | --- |

# Required Tools

- Drill with ¼” drill bit

- Wire Strippers

- Flush Cutters

- Narrow flat headed screwdriver (the screws are triangle, but a flat head should work)

- Soldering Iron

- Solder

- Ruler

- Hot glue gun

# Required Personal Protective Equipment (PPE)

- Safety Glasses

# Assembly Instructions

## Step 1

Carefully take the bubble machine out of the box without damaging it. The toy will be put back in the packaging once adapted.

## Step 2

Remove and set aside the 5 screws from the sides of the bubble blower. There are two on the other side not shown in the picture.

## Step 3

Separate the two halves of the outside of the bubble blower and set the front half aside.

## Step 4

Locate the switch.

## Step 5

Gently wiggle the switch and slide it downward until it is free.

## Step 6

Lay the bubble blower on its side.

## Step 7

Cut 2 pieces of wire 8 cm long. Use wire strippers to strip 1cm off each end of the wires.

## Step 8

Locate the two red wires. Carefully cut a small slit in the one shown above. You can peel off the rubber as needed.

## Step 9

Next, wrap one of the new wires around the newly exposed wire and solder them together. Once cooled, add tape over the joint.

## Step 10

Solder one end of your second wire to the same location as this red wire shown below.

## Step 11

Slide the switch back in the holder.

## Step 12

Insert the yellow button back into the back of the machine.

## Step 13

Reattach the back green half of the bubble machine.

## Step 14

Glue the previously existing red and black wires to the wall to keep them out of the way.

## Step 15

Reattach the purple base. Once aligned, feed the two wires through the base.

## Step 16

Solder the two wires onto the two legs of the jack closest to the mono jack hole.

## Step 17

Using a 1/4" drill bit, drill a hole in the base that can be reached by the mono jack. Cut away any loose plastic.

## Step 18

Remove the nut from the mono jack and push the mono jack through the hole. Reinstall the nut and tighten to secure.

## Step 19

Glue wires to the leg of the stand.

## Step 20

Reattach the front side of the bubble machine using the 5 screws.

## Step 21

Remove 4 screws on the back of the machine to add 4 AA batteries. Then put the screws back in.

## Step 22

Test the bubble blower. Then, repackage the device.